OMEN Alpha / issue 4

Technická dokumentace

ÚVOD
====

OMEN Alpha /Osmibitový Mikropočítač pro Elektronické Nadšence/ je jednoduchý jednodeskový počítač, postavený na procesoru Intel 8085. Jeho základní parametry jsou:

- 8085 / 80C85 CPU na frekvenci 1.8432 MHz

- 32 kB RAM

- 32 kB EEPROM

- Sériový komunikační port s rychlostí až 115.200 Bd / MC6850

- 3 paralelní porty / Intel 8255

- Aplikační sběrnice

NÁVOD K SESTAVENÍ

===============

- Zapájejte patice pro integrované obvody

- Zkontrolujte, zda spojení fungují

  - Otestujte, zda všechny piny jsou propojené jak mají

  - Proměřte, zda není zkrat mezi napájením a zemí

  - Zkontrolujte, jestli všechny integrované obvody mají přivedené napájení

- Připájejte pasivní součástky /kondenzátory, diodu, tlačítko, krystal, rezistory/

- Připojte napájecí napětí a zkontrolujte, zda vše funguje

- Vložte procesor do patice /pozor na správnou orientaci!/ a zkuste zapnout napájecí napětí. Zkontrolujte, zda oscilátor funguje / CPU vývod 37/

- Vložte do patic všechny ostatní integrované obvody. Opět: dbejte na správnou orientaci! Otočení obvodu může vést k jeho zničení.

- Připojte sériové signály TxD, RxD a GND /pinhead JP1/ k převodníku na USB. Nezapomeňte, že TxD na desce musí být připojeno k RxD na převodníku a obráceně!

- Spusťte terminálový program na PC, vyberte správný port a parametry nastavte na 115.200 Bd, 8 data bits, no parity, 1 stop bit.

- Zapněte napájení a sledujte terminál. Měl by se ohlásit Monitor

MONITOR

=======

Základní programové vybavení OMEN Alpha tvoří monitor MON85 od Dave Dunfielda, ve verzi upravené Romanem Bórikem.

Manuál najdete na https://8bt.cz/mon85 nebov Příloze 1

Můžete spustit zabudovaný TINY Basic příkazem “G1000” /bez uvozovek/ a stiskem ENTER. Z BASICu se zpět do Monitoru dostanete příkazem BYE.

DESKA PLOŠNÝCH SPOJŮ

==================

Vývody, piny a přepínače

--------------------

JP1: Sériový port. Vývody jsou GND, TxD, RxD zleva doprava, RxD je vedle nápisu JP1

JP2, JP3, JP4: Paralelní porty, viz níže. Značka označuje pin 1 - Vcc

JP5: Povolení zápisu do EEPROM. Spojení pinů 1 a 2 zakazuje zápis, spojení 2 a 3 jej povoluje. Normálně jsou propojeny 1 a 2. Pin 1 je vlevo, pin 3 vedle nápisu

JP7: Povolení vstupu RST7.5. Nechte propojené, případně zapojte rezistor 10 k

JP8: Sériové signály SID a SOD

PARALELNÍ PORTY

=============

JP2:

+-----+-----+-----+-----+-----+
| GND | PA7 | PA6 | PA5 | PA4 |
+-----+-----+-----+-----+-----+
| Vcc | PA0 | PA1 | PA2 | PA3 |
+-----+-----+-----+-----+-----+

JP3:

+-----+-----+-----+-----+-----+
| GND | PB7 | PB6 | PB5 | PB4 |
+-----+-----+-----+-----+-----+
| Vcc | PB0 | PB1 | PB2 | PB3 |
+-----+-----+-----+-----+-----+

JP4:

+-----+-----+-----+-----+-----+
| GND | PC7 | PC6 | PC5 | PC4 |
+-----+-----+-----+-----+-----+
| Vcc | PC0 | PC1 | PC2 | PC3 |
+-----+-----+-----+-----+-----+

APLIKAČNÍ KONEKTOR

=========================

Konektor je na horním okraji desky, pin 1 je označen značkou a je vlevo

Pins:

- /WR

- D0

- D1

- D2

- D3

- D4

- D5

- D6

- D7

- A0

- A1

- A2

- /RD

- IO1    --- 20h – 27h

- IO2    --- 10h – 17h

- IO3    --- 30h – 37h

- IO4    --- 08h – 0Fh              IOx signály jsou dekódovány z adresy

- IO5    --- 28h – 2Fh

- IO6    --- 18h – 1Fh

- IO7    --- 38h – 3Fh

- Vcc

- GND

- RST7.5  -- viz JP7

- CLK

- RESET  -- výstup

- READY  -- Úroveň 0 vyvolá WAIT stav

- /BUSRQ /není použito v této verzi/

- /BUSACK /není použito v této verzi /

- /není použito v této verzi /

- /není použito v této verzi /

ADRESY PERIFERIÍ
==============

ACIA 6850 – sériový port:
--------------------

DEh – Controll Register / Status Register

DFh – Data Register

PPI 8255 – paralelní port:
---------------------

04h – PORT A

05h – PORT B

06h – PORT C

07h – Control port

MAPA PAMĚTI
==========

0000h – 7FFFh  - EEPROM 32k

8000h – FFFFh – System RAM

PŘÍLOHA 1 – Monitor MON85

=========================

MON85

A

Software Debug Monitor

For the 8085/8080

Dunfield Development Systems

----------------------------

High quality tools for

Embedded Development

at low prices.

http://www.dunfield.com

Copyright 1979-2007 Dave Dunfield

All rights reserved

MON85

OBSAH

1. ÚVOD

1.1 8085 Restart Interrupts

1.2 Breakpoints

2. PŘÍKAZY MON85

2.1 A ON|OFF

2.2 B [0-7 address]

2.3 C <source> <destination> <size>

2.4 D <address>

2.5 E <address>

2.6 F <start> <end> <value>

2.7 G [address]

2.8 I <port>

2.9 L [address]

2.10 M <address>

2.11 O <port> <data>

2.12 R [rp value]

2.13 S ON|OFF

2.14 T ON|OFF

2.15 U [address]

2.16 ?

3. SEZNAM PŘÍKAZŮ

1. ÚVOD

MON85 is a ROMable interactive  debugging  program  for  the  8085

processor,  which contains a full complement of commands  to  monitor

and control the execution of your program.  It may also be used on an

8080 processor,  as long as you  do  not  attempt  to  use  the  8085

specific SIM  and  RIM  instructions,  or  the  '.5'  type  interrupt

vectors.

MON85 must be installed beginning at location $0000  in  the  8085

processor memory map, allowing it to intercept the interrupt vectors.

Provision has been made to re-vector interrupts to  locations  within

the user program.

All functions of MON85 are performed via software without hardware

assist.  The only  hardware  specific  subroutines  required  by  the

monitor are used to communicate with the console  terminal,  and  are

located at the very end of the monitor source code listing:

INIT    - Called to initialize any hardware required for I/O.

OUT     - Write the character in A to the console. No processor

registers should be modified by this routine.

IN      - Test for a character from the console, and return it

in A if one is available. Otherwise clear A to zero.

No other registers should be modified by this routine.

1.1 8085 Restart Interrupts

MON85 reserves two "restart" interrupts:  RST 0 is similar to a

physical RESET,  causes  a  cold  restart  of  the  monitor.  This

instruction should be used with CAUTION during a debugging session

because  any  breakpoints  set  in  the  user  program   will   be

"forgotten",  and not properly removed.  RST 1 is used by MON85 to

regain   control   at   breakpoints.   Opcodes   of   breakpointed

instructions are replaced by RST 1 whenever the  user  program  is

executed,  and are restored whenever the  monitor  is  re-entered.

This insures that the operation of breakpoints is  transparent  to

you during the debugging session.

If for any reason a 'RST 1' ($CF) instruction is encountered in

the user program  (and is not a breakpoint),  command mode will be

entered without the '** Breakpoint' message.

All  other  "restart"  interrupts  are   re-vectored   to   the

corresponding locations in the first page of  memory  occupied  by

the user program (as identified by the 'U' command).

1.2 Breakpoints

MON85 allows you to set breakpoints in your program,  such that

you  will  be  given  control  whenever  the  program  reaches   a

breakpoint,  and can examine or change things  before  proceeding.

MON85 also allows you to TRACE a program, so that you can see each

instruction and register contents as it executes.

MON85 is completely transparent to  the  program  being  tested

(unless timing loops are interrupted with BREAKPOINTS  or  TRACE).

When a breakpoint is encountered,  or an  instruction  is  traced,

MON85 uses one stack entry on the user program stack.  Since  this

consists of a PUSH and a POP, it should not affect any information

stacked by the user program. However, you should be aware of this,

in case you are examining the  stack,  or  your  program  trys  to

reclaim data already popped from  the  stack.  (It  is  very  poor

practice,  to write programs which depend on  the  stack  contents

below the stack pointer).

When a breakpoint is encountered, the message '** Breakpoint n'

is printed where n is the number of the breakpoint (0-7). If TRACE

is ON, no other action is taken, otherwise command mode is entered

(If the  'A'  switch  (see below)  is ON the  registers  are  also

displayed).

2. MON85 COMMANDS

The following commands are recognized by MON85, and may be entered

whenever MON85 is in COMMAND MODE,  which is indicated by the  'C>  '

prompt:

2.1 A ON|OFF

This switch turns ON or OFF the automatic register  display  which

occurs whenever a breakpoint is encountered or an  instruction  is

stepped in trace mode  (see below).  The default value for 'A'  is

ON.

2.2 B [0-7 address]

Sets one of eight breakpoints [0-7] at the specified address. Once

set, a breakpoint remains in effect until it is removed by setting

it to address ZERO (0).  Breakpoints are completely invisible, and

may be added,  removed or changed  at  any  time  without  adverse

affects.  If the  'B'  command is issued  with  no  operands,  the

current breakpoints,  and the settings of the  'A',  'S'  and  'T'

flags  (see below)  are displayed.  A displayed address of  '****'

indicates that a breakpoint is not set.

2.3 C <zdroj> <cíl> <počet>

Kopíruje  <počet>  bytů paměti z místa daného adresou <zdroj> do oblasti 
          od adresy <cíl>.

2.4 D <adresa>

Displays memory in assembly listing format, starting at <address>,

continuing until an ESCAPE character is  entered.  Output  can  be

temporarily stopped with the SPACE BAR,  and  restarted  with  the

RETURN key  (Additional SPACE's will  output  single  lines).  The

output is displayed in the following form:

'<address> <opcodes> <ascii> <instruction>'

<address>  is  the  current  memory  location,  <opcodes>  is  the

instruction  opcodes  (1,  2  or  3  bytes),  <ascii>   is   ASCII

representation of  <opcodes>  (all  non-printable  characters  are

displayed as dots),  and  <instruction>  is the assembly  language

instruction and operands which <opcodes> represents.

2.5 E <address>

Edits memory, starting at <address>. The address, and its contents

are displayed, followed by a '=' prompt. Sub commands are:

nn [nn ...] - Replace memory contents with HEX data bytes

'text ...   - Replace memory contents with ASCII text

-           - Backup to previous locations

<blank line>- Advance to next location with change

<ESCAPE>    - Return to MON85 command prompt

2.6 F <start> <end> <value>

Fills memory between the  <start>  and  <end>  addressses with the

specified <value>.

2.7 G [address]

Loads the user program registers,  and  begins  execution  at  the

specified address.  If no [address] is given,  execution begins at

the address contained in the user program program counter (PC).  A

simple  'G'  with no operands,  is all that is  needed  to  resume

execution after a breakpoint interrupt,  or to resume  trace  mode

exection.

2.8 I <port>

Reads the specified 8085 I/O port, and displays the data.

2.9 L [address]

Downloads code from the console port.  MON85  will  recognize  and

accept either INTEL or MOTOROLA hex format download  records.  The

address in the FIRST download record is recorded  as  the  program

BASE address  (See 'U' command).  If an  [address]  is given,  the

address fields of each record are adjusted so that the code  image

will be loaded into memory beginning at that address, otherwise it

is placed in memory at the  absolute  address  contained  in  each

record.

If you accidently enter this command, you may enter either 'S9' or

':00'  to signify a null download file and return to  the  command

prompt.

2.10 M <address>

Displays memory in HEX/ASCII dump format starting at the specified

address. ESCAPE, SPACE, and CARRIAGE-RETURN can be used to control

the listing, the same is with the 'D' command.

2.11 O <port> <data>

This command writes the  <data>  byte to the  specified  8085  I/O

<port>.

2.12 R [rp value]

Changes the contents of the user program register pair <rp> to the

specified <value>. Valid pairs are BC, DE, HL, SP, PC, and PSW. If

no operands are given,  the contents of the user program registers

are displayed.

2.13 S ON|OFF

Controls the handling of subroutine calls when in trace mode  (see

below).  When 'S' is set ON,  subroutines will be be traced in the

normal fashion.  When set OFF,  subroutines calls are not  traced,

and trace will resume at the next instruction following  the  CALL

(After the subroutine executes).

WARNING: DO NOT SET BREAKPOINTS IN THE SUBROUTINES WHEN 'S' IS SET

OFF.  When 'S' is set off, MON85 considers the subroutine (and all

of it's embedded instructions)  to be one single operation.  If  a

breakpoint is encountered inside the subroutine,  TRACE will  lose

the address at which to resume following the subroutine call. This

will cause  unpredictable  action  when  the  subroutine  returns.

Otherwise,  'S'  may be turned on and off  at  any  time  with  no

adverse effects. The default value for 'S' is ON.

2.14 T ON|OFF

Turns TRACE mode ON and OFF.  When TRACE is set  ON,  and  the  GO

command is issued,  MON85  will  first  prompt  with  'T>'  before

beginning program execution.

Entering a space  (' ')  will display and execute one instruction,

and return to the 'T>' prompt.

Entering  '?'  will display the processor registers.  (If the  'A'

switch is set ON,  they will always be displayed  following  every

instruction that is traced).

An ESCAPE charecter will  cause  MON85  to  return  to  the  usual

command prompt  (TRACE remains ON,  and will resume with the  next

'G' command).

Trace may be turned on and off at any time in a program,  with  no

adverse effects.  (If you begin execution with TRACE off, you will

have to hit a breakpoint to get you back to  command  mode  before

you can turn TRACE on).

When a breakpointed instruction is encountered by the tracer,  The

message  '** Breakpoint n'  will be  issued  at  the  end  of  the

previous instruction, indicating that the breakpointed instruction

occurs NEXT.  Pressing  the  SPACE  BAR  would  then  execute  the

breakpointed instruction. The default value for 'T' is OFF.

2.15 U [address]

Identifies the starting address of a user program. MON85 uses this

address to re-map the "restart"  interrupt vectors.  When a 'L'oad

command  is  performed,  MON85  initializes  'U'  to  the  address

contained in  the  first  download  record.  If  no  [address]  is

specified,  MON85 will display the current user  program  starting

address.

2.16 ?

This command displays a short summary of the other MON85 commnds.

3. COMMAND SUMMARY

A ON/OFF                - Enables/Disables auto register display.

B [0-7 address]         - Sets/Removes/Displays breakpoints.

C <src> <dest> <size>   - Copy memory

D <address>             - Displays memory in disassembly format.

E <address>             - Edit memory contents.

F <start> <end> <value> - Fill memory with a value.

G [address]             - Begin/Resume program execution.

I <port>                - Input from port

L [address]             - Download from host

M <address>             - Displays memory in dump format.

O <port> <data>         - Output to port

R [rp value]            - Sets/Displays register contents.

S ON|OFF                - Enables/Disables subroutine traceing.

T ON|OFF                - Enables/Disables TRACE mode.

U [address]             - Set/Displays program base address.

?                       - Display command summary

MON85 Improvements by Martin Borik:

- Support for undocumented 8085 instructions DSUB B, ARHL, RDEL, LDHI d8, LDSI d8, LHLX D, SHLX D, JNK a16, JK a16, RSTV

- Command R displays all flags of F register (SZKA3PVC). If flag is not set dash '-' is displayed.

- Added restart vector RST 8 (0040h) for possibility to handle RSTV call.

- Changed TRACE mode. After entering TRACE mode, instruction on actual PC and content of registers (if it is switched on) are displayed. Entering a space ' ' executes this instruction, and returns to the 'T>' prompt with the next instruction.

- Instructions LXI, DAD, INX, DCX displays argument 'SP' rather than 'S'.

- Commands that requires 1 byte parameter raises error if entered value not fit to 1 byte.

- Command 'C' checks overlap of source and destination block and for copying uses appropriate direction.

- Command 'F' checks <start> and <end> parameters and raises error, if <end> is lower than <start>.

- Added command 'H' to send out memory content in Intel HEX format.

- Sending of LF and CR characters were reversed and are sent in the usual order - CR first and followed by LF.